Shuttle Portable Computer Flight Procedures Handbook

This is an edited version of the official Shuttle Portable Computer (SPoC)

Flight Procedures Handbook.  This document applies only to the version of

SPoC available through NASA Spacelink.

Orbital state vectors required for this program are available for each

Shuttle mission from NASA Spacelink (Data line 205/895-0028).  Use the

"M50 Elements" figures.  And remember that SPoC requires figures

in thousands of feet.  The M50 elements on NASA Spacelink are often

given in feet.  Don't forget to move the decimal three places to the

left before entering the figures into SPoC.

The following hardware is required for proper operation of this program:

IBM PC or compatible with math co-processor installed

Hard Drive at location C:

Minimum 512K available memory  NOTE:  If you are running software

(network software for example) that occupies enough memory to

leave less than 512K for SPoC, SPoC will very likely crash.

GRID.EXE, SPOC.BAT, and the PROGRAMS subdirectory, must be located in the

Drive C: ROOT directory.

To run the SPoC software type SPoC at the C> prompt.

NOTE: The CODE key referred to in this document is the ALT key on IBM PC

compatible computers.

NOTE: As you will read in the following document, SPoC was written to

run on  a GRID computer--not an IBM-PC.  Before making SPoC available

through NASA Spacelink, we tested it on a variety of IBM-PC and

compatible computer configurations with mixed results.  For example,

SPoC seemed to run properly on one computer, but its VGA display

scrambled the screen making the program unreadable.  Other VGA

displays gave no problems.  So if SPoC doesn't run on your

particular setup, try it on another computer.

SHUTTLE PORTABLE COMPUTER (SPoC)

FLIGHT PROCEDURES HANDBOOK

NATIONAL AERONAUTICS AND SPACE ADMINISTRATION

LYNDON B. JOHNSON SPACE CENTER

HOUSTON, TEXAS

TABLE OF CONTENTS

SECTION      TITLE

1.0          INTRODUCTION

1.1          SPoC Mission Definition

1.2          How To Use This Handbook

2.0          HARDWARE

3.0          SPoC USER INTERFACE

3.1          Modifier and Command Keys

3.2          Special Key Combinations

3.3          The Selection Box

3.4          SPoC Menus

3.5          SPoC Forms

3.6          Messages

4.0          OVERVIEW OF SPoC PROGRAMS

5.0          BOOT UP SCREEN

6.0          LAUNCH TIME UPDATE

7.0          GMT UPDATE

8.0          STATE VECTOR UPDATE

9.0          SPoC MAIN MENU

10.0         WORLD MAP

10.1         World Map Help Menu

10.2         World Map Display

10.2.1       Current MET

10.2.2       AOS/LOS of TDRS Coverage

10.2.3       AOS/LOS of Ground Communication Site

10.2.4       Spacecraft Sunrise, Noon, Sunset

10.2.5       TDRS Coverage

10.2.6       Earth Day/Night

10.2.7       Earth Observation Area

10.2.8       Ground Communication Sites

10.2.9       Shuttle Bug

10.2.10      Enlarged F-Stop Display

10.2.11      Current and Two Predicted Ground Tracks

10.2.12      Tone Indicator

10.2.13      Latitude, Longitude and Current F-Stop

10.2.14      Ascending Node Time

10.2.15      Earth Observation Site

10.2.16      Longitude, MET, and Countdown Timer For Next Ascending Node

10.2.17      Longitude, MET, and Time Since Previous Ascending Node

10.2.18      Sun Glint Angle and Azimuth

10.2.19      Sun Glintpoint Area

10.3         PSAT

10.4         SITE MANAGER

10.4.1       SITE MANAGER Exit Options

10.4.1.1     CODE-D   INVOKE DEFAULT COMMUNICATION SITES

10.4.1.2     CODE-S   SAVE CHANGES PROCEED TO DAYMAP

10.4.1.3     CODE-ESC   NO CHANGES PROCEED TO DAYMAP

10.4.2       SITE MANAGER Functions

10.4.2.1     CODE-N   SELECT COMMUNICATION NETWORK

10.4.2.2     CODE-I   SELECT INDIVIDUAL COMMUNICATION SITES

10.4.2.3     CODE-P   ADD COMMUNICATION SITE

10.4.2.3.1   Adding a Ground Communications Site

10.4.2.3.2   Adding a TDRS

10.4.2.3.3   Editing User Input Ground Site Data

10.4.2.3.4   Editing User Input TDRS Data

10.4.2.4     CODE-X   DELETE COMMUNICATION SITES

10.4.2.5     CODE-CTRL-SHIFT-P   ADD EARTH OBSERVATION SITE

10.4.2.5.1   Adding An Earth Observation Site

10.4.2.5.2   Editing User Input Earth Observation Site Data

10.4.2.6     CODE-CTRL-SHIFT-X   DELETE EARTH OBSERVATION SITES

APPENDIX     TITLE

A            INITIALIZING THE SPoC

B            WORLD MAP DEMONSTRATION

E            SPoC HISTORY

F            GLOSSARY

SECTION 1.0  INTRODUCTION

The concept of an independent computational capability aboard the Shuttle

has been operational since STS-1.  Originally the HP41 hand-held

calculator was employed for this purpose, but it has significant

limitations.  The SPoC greatly augments HP41 capabilities.  STS-9 was the

first Shuttle mission manifested with a SPoC, and it proved to be such an

asset for the crew that a SPoC has been manifested on every subsequent

Shuttle mission.  In its original role it was used as a situational

awareness aid, depicting the Orbiter and its ground track over the earth's

surface.  The SPoC has evolved to provide a backup alternative means of

obtaining landing opportunities, deorbit targets, and center of gravity

management procedures.  These targets and procedures are only utilized in

the event of a total loss of communications with the ground or a time

critical systems failure during a loss of communication period requiring

an emergency deorbit before ground assistance can be provided.  Nominally,

the SPoC is unstowed and set up during the post insertion time frame and

remains active until entry interface minus 20 minutes.

The SPoC saves Mission Control Center (MCC) personnel time and resources

by eliminating need to routinely transmit such data as Aquisition of

Signal/Loss of Signal (AOS/LOS) times via voice or teleprinter.  During

the mission, the Flight Dynamics Officer (FDO) will take SPoC system

questions in the MCC Mission Operations Control Room (MOCR) and pass them

to the SPoC Team if necessary.

1.1 SPoC Mission Definition

The SPoC is a portable "lap top" microcomputer providing the crew and

mission operations personnel with an independent computational and display

facility, enhancing productivity and reliability.  This microcomputer is

mandatory for flight because of the crew support it provides.  The SPoC

was designated as noncritical hardware by Configuration Control Board

Directive 1253 in 1983.  This classification was reaffirmed in 1987 by

On-Orbit Flight Techniques Panel Meeting #82 [ref. NASA memo DA8-88-04

(FT)].  Currently, there are four major roles for the SPoC.

a) Provide automated cues for such events as upcoming ground communication

times, earth observation site availability, sunrise and sunset times,

nodal  crossings, orbiter ground track, and location over the earth's

surface.

b) Provide a backup alternative means of obtaining landing opportunities,

deorbit targets, and center of gravity management procedures in the event

of a total loss of communications with the ground or a time-critical

systems failure during a loss of communication period requiring an

emergency deorbit before ground assistance can be provided.

c) Provide computational and display support for special purpose

applications on a flight-specific basis (e.g. the Orbit Refueling System

{ORS} and the Gravity Gradient Free Drift programs).

d) Provide mission status and contingency deorbit support in the Emergency

Mission Control Center (EMCC).

1.2 How To Use This Handbook

This handbook relates procedures and concepts associated with SPoC

operation.  Acronyms and other terms pertinent to this discussion, but

whose definitions may not be apparent from context, are defined in

APPENDIX F (GLOSSARY).  As a convention, the names of SPoC applications

will be in capitals.  A brief history of the SPoC is presented in Appendix

E.

If you are interested in operating SPoC immediately, proceed to

Appendix A, INITIALIZING THE SPoC, for your first SPoC session.

Appendices A through D are designed to provide an introductory overview

without exercising all SPoC capabilities.  For the location of complete

and detailed information, consult the Table of Contents.

SECTION 2.0  HARDWARE

The SPoC has 384k bytes of nonvolatile bubble memory for use in storing

programs and data.  Four Erasable Programmable Read Only Memory (EPROM)

slots, located above the keyboard, are user-accessible.  Each SPoC EPROM

has a 64k byte capacity, providing 256k bytes of Read Only Memory (ROM).

The SPoC directly accesses 512k bytes of Random Access Memory (RAM).  The

foldup display is a high resolution electroluminescent flat panel which

measures 7-and-9/16 by 3-and-3/4 inches (512 pixels by 256 scanlines).

There are four types of keys on the SPoC: alphabetic/numeric keys,

modifier keys for issuing commands, arrow keys for positioning the cursor,

and a return key for selection box control.  The memory, display, and

57-character keyboard are in a magnesium case,  which together weighs

approximately 10 pounds.  The unit will fit into one-half of a standard

size briefcase.

SPoC is hosted on a GRiD Compass computer purchased off-the-shelf and

modified for flight.  A standard lithium battery circuit that provides

power for the internal clock is replaced by a silver oxide battery and

equipped with diodes and a fuse to ensure fail-safe operation. The

internal modem is removed.  Flight displays are fitted with

electromagnetic interference filters.  This prevents electromagnetic

interference generated by the electroluminescent display from harming the

General Purpose Computers (GPCs) or other equipment onboard.  When used

onboard the Shuttle, SPoC runs on phase A ac power and uses a power cable

which is compatible with the Orbiter's power outlets.  Finally, the foot

of a flight SPoC is milled to permit fan exhaust flow when folded up for

flight operation.

SECTION 3.0  SPoC USER INTERFACE

SPoC applications have been designed for similarity in appearance and

operation.  Utilization of menus and forms allows the user to select,

input, edit, and execute without unnecessary typing.  Menus permit the

user to select a single item for execution, while forms permit the user to

input several data values on one display.  Prompts appear at the bottom of

the screen, as an aid to the user.  When appropriate, error messages also

appear at the bottom of the screen instructing the user how to correct

erroneous inputs.

NOTE: Messages are displayed on the screen in inverse video.  In this

document, messages displayed on screen images will appear as black

characters against a white background.  This is done for readability.

"3.1 Modifier and Command Keys"

Modifier keys include the "CODE" keys, "SHIFT" keys, and the "CONTROL"

key.  The "CODE" keys and "SHIFT" keys are located at the lower left and

lower right of the keyboard.  The "CONTROL" key is located at the lower

left corner of the keyboard.  The "RETURN" key, "ESCAPE" key, and any

other keys that invoke a menu option are command keys.  The most often

used modifier keys are the "CODE" keys. To execute any menu option or to

accept the settings in a form, the user must hold down a modifier key(s)

and then press the appropriate command key.

"CAUTION": SPoC EXPECTS THE COMMAND KEY "PRESS" ACTION TO BE BRIEF (NO

MORE THAN 0.2 SECONDS).  Pressing a command key for finite intervals may

issue multiple instructions with unpredictable results.

"3.2 Special Key Combinations"

Throughout the remainder of this handbook, the following convention will

be followed.  "CODE-[key]" will indicate that the "CODE" key is held down

until the "[key]" is pressed.  As an example, "CODE-RETURN" indicates that

the user holds down the "CODE" key until the "RETURN" key is pressed.

"CODE-ESCAPE" indicates that the "CODE" key is held down until the

"ESCAPE" key is pressed.  "CODE-M" indicates that the "CODE" key is held

down until ""M"" is pressed.  "CODE-CTRL-SHIFT-N" indicates that the

"CODE" key and the "CTRL" key are held down until ""N"" is pressed.  A

menu item will be executed or form settings will be accepted only if a

"CODE-[key]" combination is pressed.  The "CODE-RETURN" combination

executes the menu option selected or accepts the form settings selected.

The "CODE-ESCAPE" combination has a special function.  To exit a menu

without executing any of the displayed options or to exit a form without

modifying the form settings, press "CODE-ESCAPE".  If the user presses

"CODE-ESCAPE", program control returns to the previously displayed screen.

If "CODE-ESCAPE" is pressed at the main menu of a program, the program is

exited and control returns to the SPoC Main Menu, which displays the

available applications.  A "CODE-ESC" from the SPoC Main Menu is ignored.

"3.3 The Selection Box"

The selection box is a moving rectangular outline that encloses the

current option or setting.  In a menu, the selection box identifies the

option that will be executed when the user presses "CODE-RETURN".  In a

form, the selection box identifies data currently being modified.

Movement of the selection box is controlled by the "RETURN" key.  Each

time the "RETURN" key is pressed, the selection box advances to the next

option.  If the "RETURN" key is pressed while the selection box is located

on the last option of the menu, the selection box will advance to the

first option on the menu.

If the menu in receives a "CODE-RETURN" with the selection box located on

the first option, the WORLD MAP application will be executed.  Press the

"RETURN" key once, and the selection box advances to the next option.

If the menu receives a "CODE-RETURN" with the selection box located on the

second option, STATE VECTOR UPDATE will be executed.

"3.4 SPoC Menus"

SPoC menus permit the user to select one option to be executed with a

minimum of keystrokes.  To select an option, press the "RETURN" key

repeatedly until the selection box is located on the option to be

executed.  Press "CODE-RETURN" to execute the option.  There is an

alternate method of selecting and executing an option from a menu.  Press

"CODE-[key]", where "[key]" is the designated letter for the option to be

executed.  Using the SPoC Main Menu as an example, DEORBIT can be selected

by pressing the "RETURN" key three times to locate the selection box on

the menu option DEORBIT.  DEORBIT is then executed by pressing

"CODE-RETURN".  Using the alternate method, DEORBIT can be selected and

executed by pressing "CODE-D" regardless of where the selection box is

currently located.

"3.5 SPoC Forms"

SPoC forms permit the user to enter or edit data for one or more items.

Forms are different from menus for these reasons:

a) Forms let users change the settings of several items.  Menus let users

select only one option.

b) Users may input their own settings to forms.  Many forms do not limit

users

to predefined choices.

c) When users press "CODE-RETURN", they accept the settings of all form

items,

not just the outlined setting.

A form consists of the following components.

"Items": labels which identify data to be modified.  Each item has a

setting associated with it.

"Settings": actual values the user has typed, chosen from the choice band,

or received by default.  These settings are either choice or editable

fields, depending on the application.

"Selection box": moving indicator that surrounds the setting currently

being modified.  "RETURN" moves it down and "SHIFT-UPARROW" moves it up.

"Choice band": located at the top of the form, it may be configured in one

of three ways.  If there are choices for the current item, the choices are

displayed there.  If the item is an editable item, the choice band may be

blank or contain pertinent information with the edit prompt.

"Choices": predefined values for a setting, which appear in the choice

band.  The highlighted choice appears within the selection box

automatically.

"Highlighted box": indicates the choice that appears in the selection box,

when choices are available.  Pressing the arrow keys moves the highlighted

box among choices.  Horizontally grouped choices utilize the "LEFTARROW"

and "RIGHTARROW" keys for movement, while vertically grouped choices are

selected with the "UPARROW" and "DOWNARROW" keys.  If the item is an

editable item, the highlighted box, if displayed, will contain pertinent

information and the edit prompt.

A setting is editable if there are no choices listed in the choice band.

In this case, the edit prompt appears in the selection box as a small

flashing triangular field.  A setting is a choice setting if there are

choices listed in the choice band.

To change data for an editable setting, use the "LEFTARROW" and

"RIGHTARROW" keys to locate the flashing edit prompt.  The following

actions are possible on all data left of the prompt.

1) Press the "BACK SPACE" key to erase the rightmost character of data.

2) Press "CODE-BACK SPACE" to erase the rightmost word of data.

3) Press "CODE-SHIFT-BACK SPACE" to erase all data.

Data may be inserted right of the flashing prompt by pressing the

appropriate keys.  Once satisfied with the settings, press "CODE-RETURN"

to incorporate all settings and proceed to the next form or menu.  To exit

a form without accepting any new settings, press "CODE-ESC".  Upon

pressing "CODE-ESC", the former item settings are restored and the

previous form or menu will be displayed.

"3.6 Messages"

There are three types of SPoC messages.

1) Prompts

2) Error messages

3) Informative messages

As an aid to the user, each SPoC menu or form has prompts displayed at the

bottom of the screen.  Information provided by prompts will inform the

user what action should be taken next.

Error messages provide information for correcting erroneous user inputs.

Informative messages provide status on what action is currently being

taken.  Informative messages require no user action.

SECTION 4.0  OVERVIEW OF SPoC PROGRAMS

In this section, a high level overview of SPoC software will be presented.

There are five operating modes for SPoC software: InFlight, Debug,

Development, SmartDemo, and DummyDemo.

This document pertains only to the InFlight mode.

"LAUNCH TIME UPDATE"

During SPoC initialization, the actual launch Greenwich Mean Time (GMT)

can be input to update the launch time expected premission.  The launch

time is stored in bubble memory and used to compute Mission Elapsed Time

(MET).

"GMT UPDATE"

Current GMT can be updated in SPoC's internal clock.  The internal clock

is maintained by an internal battery during periods when SPoC power is

OFF.

"STATE VECTOR UPDATE"

This program allows the user to input position and velocity vectors in the

Mean of 50 (M50) coordinate system.  These vectors are obtained from the

GPC Memory Page and are written to bubble memory for access by other

programs.

"SPoC EXECUTIVE"

The SPoC EXECUTIVE program displays the SPoC Main Menu from which the user

may select the application desired.  Upon exiting any program selected

from the SPoC Main Menu, program control returns to the SPoC Main Menu.

"WORLD MAP"

The SPoC presently runs WORLD MAP as its primary program.  A graphics

display of a world map is overlaid by the Orbiter's current trajectory

status.  The following information/functions are available.

Current MET

Fixed coastline and Spaceflight Tracking and Data Network (STDN)/

Tracking and Data Relay Satellite (TDRS) coverage graphics

Day and night areas on the earth's surface

AOS/LOS and identification of next/current ground communication site

AOS/LOS and identification of next/current TDRS coverage

Current and two predicted ground tracks

Current position of the Orbiter on the world map

AOS of the next earth observation site

AOS of future earth observation sites

Current F-stop

F-stop for the selected earth observation site

Predicted Site Acquisition Table (PSAT) Display

Table of TDRSS AOS/LOS

Communication/Earth Observation site reconfiguration

Latitude and longitude of the Orbiter

Time to Orbiter sunrise, noon, or sunset

Sun glint position on the world map

Sun glint look angles

Longitude and MET of the next/previous ascending node

Time until/from the next/previous ascending node

SECTION 5.0  BOOT UP SCREEN

When SPoC is turned ON, the Boot Up Screen is displayed after several

seconds.  At the bottom center of this screen will be displayed the

message:

Boot in Progress

Please Wait

The contents of the rest of the screen will indicate:

1) Specific or Generic Flight Load

2) Flight or Training Load

3) Version Number

After the operating system has been loaded, the Boot Up Screen will be

erased.  The following message will be displayed at the bottom of the

screen while SPoC EXECUTIVE searches for the necessary applications.

"Searching for all the necessary SPoC programs"

Next, SPoC EXECUTIVE will search for necessary data files.  The following

message will be displayed.

"Searching for all the necessary SPoC data files"

Upon completion of the data file search, the LAUNCH TIME UPDATE form will

be displayed.  If this form does not appear, a fatal error has been

detected in the current SPoC software configuration by SPoC EXECUTIVE.

SECTION 6.0  LAUNCH TIME UPDATE

This program allows the user to update the GMT of launch.  Upon completion

of the boot cycle, the LAUNCH TIME UPDATE form (FIGURE 6.1) is displayed.

Initial settings displayed indicate nominal launch time.  These settings

can be updated by the user in the event of a launch delay.

If the launch form settings are correct, press "CODE-RETURN".  Otherwise,

correct any erroneous data.

Acceptable inputs for LAUNCH TIME UPDATE FORM:

YEAR    - 1983 through 1999

DAY     - 1 through 365 in common years, 1 through 366 in leap years

HOUR    - 0 through 23

MINUTE  - 0 through 59

SECOND  - 0 through 59

Possible Error Messages:

1) "ERROR: Time parameter(s) input incorrectly"

Problem:  One or more of the settings are not within acceptable limits

or are not formatted as integers.

Solution: The symbol "->" will appear next to out-of-range or

improperly formatted settings.  Replace erroneous data with good data.

SECTION 7.0  GMT UPDATE

After pressing "CODE-RETURN" from the LAUNCH TIME UPDATE form, the GMT

UPDATE form will automatically appear.  This form can also be accessed

from the SPoC Main Menu.  The time displayed in this form is used to set

the internal SPoC clock when "CODE-RETURN" is pressed.  The GMT will have

to be updated to correct internal clock drift.  SPoC clock drift will

average approximately five seconds per day.

Acceptable inputs for GMT UPDATE form:

DAY    - 1 through 365 in common years, 1 through 366 in leap years

HOUR   - 1 through 23

MINUTE - 1 through 59

YEAR   - 1980 through 1999

Possible Error Messages:

1) "ERROR: Time parameter( s ) input incorrectly"

Problem: The settings for the items listed in the error message are not

within acceptable limits.

Solution: The symbol "->" will appear next to out-of-range settings.

Replace data that are not within acceptable limits.

Possible Caution Messages:

1) "CAUTION: Vector more than 2 days old"

Problem: SPoC GMT and the current SPoC state vector time tag are

incompatible.  The state vector time tag is two days older than

current SPoC GMT.

Solution: If GMT settings are incorrect, press "CODE-ESC" to

reinitialize the form.  Correct erroneous data.

If GMT settings are correct, press "CODE-RETURN".  The state

vector time tag settings can be updated in the STATE VECTOR UPDATE

form.

2) "CAUTION: Vector is in the future"

Problem: SPoC GMT and the current SPoC state vector time tag are

incompatible. The time tag for the current SPoC state vector is

in the future.

Solution: If GMT settings are incorrect, press "CODE-ESC" to

reinitialize the form.  Correct erroneous data.

If GMT settings are correct, press "CODE-RETURN".  The state

vector time tag settings can be updated in the STATE VECTOR UPDATE

form.

SECTION 8.0  STATE VECTOR UPDATE

The STATE VECTOR UPDATE form allows users to update the SPoC state vector.

This form will automatically appear after pressing "CODE-RETURN" from the

GMT UPDATE form during the SPoC initialization sequence.  STATE VECTOR

UPDATE can also be accessed from the SPoC Main Menu.

Components of the previously input time tag and state vector are initially

displayed in the STATE VECTOR UPDATE form.  Prelaunch, a predicted nominal

post-OMS-2 state vector is loaded into the SPoC.

In OPS 2, components of the current state vector are provided by the GPCs

using the GNC SPEC 34 Orbit Targeting display.  Remember, values displayed

on the CRT are continuously updated.  Press SPEC to freeze the display.

The time tag, along with position and velocity components, can now be

entered into the SPoC STATE VECTOR UPDATE form.

In OPS 3, components of the current state vector are provided by the GPCs

using the GNC SPEC 0 GPC Memory display.  SPEC 0 item entries are listed

in the Reference Data Book Flight Supplement.  After the six state vector

components have been requested and displayed, press SPEC to freeze the

display.  The time tag, along with position and velocity components, can

now be entered into the SPoC STATE VECTOR UPDATE form.  Note: The GPC time

tag displayed with the state vector may have up to a 1.8 second delta from

the state vector's true time tag.  This is due to differing update rates

for these quantities on the GPC Memory display.

When settings in the form reflect the frozen SPEC 34 (SPEC 0 in OPS 3),

press "CODE-RETURN".  The inclination, HA, and HP are displayed.  This

serves as a data check for state vector inputs.  If values displayed are

reasonable, press "CODE-RETURN" and the Main Menu will be displayed.

If values are not reasonable, press "CODE-ESC" and the STATE VECTOR UPDATE

form will be reinitialized.  Correct any erroneous state vector settings

and press "CODE-RETURN".  Again the inclination, HA, and HP will be

displayed.  If the values displayed are reasonable, press "CODE-RETURN".

Otherwise, press "CODE-ESC" to reinitialize the form.

Acceptable inputs for STATE VECTOR UPDATE time tag:

DAY (GMT)    - 1 through 366

HOUR (GMT)   - 0 through 23

MINUTE (GMT) - 0 through 59

SECOND (GMT) - 0 through 59

Possible error messages:

1) "ERROR: Time parameter( s ) input incorrectly"

Problem: Settings for the GMT items listed in the error message are not

within acceptable limits or are not formatted as integers.

Solution: The symbol "->" will appear next to out-of-range settings.

Replace data that are not within acceptable limits.

Possible caution messages:

1) "CAUTION: VECTOR MORE THAN 2 DAYS OLD"

Problem: The SPoC GMT and the current SPoC state vector time tag are

incompatible.  The time tag is more than two days old

based on the current SPoC MET.

Solution: If the state vector time tag settings are incorrect, press

"CODE-ESC" to reinitialize the form.  Correct erroneous data.

If the state vector time tag settings are correct, press

"CODE-RETURN".  GMT settings can be updated in the GMT UPDATE

form.

2) "VECTOR IS IN THE FUTURE"

Problem: The SPoC GMT and the current SPoC state vector time tag are

incompatible.  The time tag is in the future based on

the current SPoC MET.

Solution: If the state vector time tag settings are incorrect, press

"CODE-ESC" to reinitialize the form.  Correct erroneous data.

If the state vector time tag settings are correct, press

"CODE-RETURN".  GMT settings can be updated in the GMT UPDATE

form.

SECTION 9.0  SPoC MAIN MENU

The Main Menu allows users to invoke SPoC applications.  After the SPoC

initialization sequence has been completed, the SPoC Main Menu will be

displayed.  This menu will also be displayed upon exiting any SPoC

application.

Once the user has executed a menu option to select an application, status

messages will appear at the bottom of the screen.  These messages provide

status on files being loaded or any processing in progress.

Possible error message:

"STATE VECTOR TIME IS INCOMPATIBLE WITH GMT"

Problem: SPoC current MET and the SPoC state vector MET time tag are

incompatible.  The time tag for the current SPoC state vector is

either in the future, or two days older than the current SPoC MET.

Solution:

A) Press any key and return to the SPoC Main Menu.

B) If the displayed MET is correct, select STATE VECTOR UPDATE

from the SPoC Main Menu.  Otherwise, proceed to Step D.

C) Verify that state vector time tag settings are correct.

1) If time tag settings are incorrect, correct erroneous

settings, press "CODE-RETURN", and return to the SPoC Main

Menu.

2) If time tag settings are correct, press "CODE-ESC" to

return to the SPoC Main Menu without changing form

settings.

3) Proceed to Step F.

D) Select SET GMT from the Main Menu.

E) Verify that GMT settings are correct.

1) If GMT settings are incorrect, correct erroneous

settings and return to the SPoC Main Menu.

2) If GMT settings are correct, press "CODE-ESC" to return

to the SPoC Main Menu.

F) Make selection from the Main Menu.

SECTION 10.0  WORLD MAP

SITE MANAGER Main Menu

The first time WORLD MAP is selected from the SPoC Main Menu, the SITE

MANAGER Main Menu will be displayed.  SITE MANAGER is a WORLD MAP function

which will be detailed in Section 10.4.  For now, execute the first menu

item "INVOKE DEFAULT COMMUNICATION SITES" and proceed to the WORLD MAP

display.

WORLD MAP serves as a crew aid which provides a situation display, various

timers, and tables.  The situation display includes a map with a longitude

range from 180 degrees West to 180 degrees East, and a typical latitude

range from 65 degrees North to 65 degrees South.  A map with any valid

latitude range can be provided by the SPoC Team upon request.

10.1 World Map Help Menu

The WORLD MAP Help Menu may be invoked by pressing "CODE-?" with the WORLD

MAP display active.  This menu provides the "CODE-[key]" combinations

associated with each WORLD MAP function.  WORLD MAP functions may be

invoked from the Help Menu, or may be invoked from the WORLD MAP display

directly with the proper "CODE-[key]" input.  To return to the WORLD MAP

Display, press "CODE-ESC".

10.2 World Map Display

The contents of the WORLD MAP Display are described in detail below.

Following is a rough drawing of the World Map Display screen.  The numbers in

parentheses represent the ITEMS described in the text below.

+==========================================================+

|   (1) |       |  (2)     | (3)     |             | (4)   |

|----------------------------------------------------------|

|                                                          |

|                                                          |

|                                                          |

|             [Picture of earth showing day and night,     |

|                  orbital paths and current location of   |

|                  shuttle, earth-based tracking station   |

|                  locations and coverage areas, earth     |

|                  observation opportunities.]             |

|                                                          |

|                                                          |

|                               +-----+                    |

|                               |     |                    |

|                               | (10)|                    |

|(12)                           |     |                    |

|----------------------------------------------------------|

|  (15) |           |    (13,16,17,18) |        |(14,16,17)|

+==========================================================+

10.2.1 Current MET (Item 1)

DD/HH:MM:SS fields will update approximately every second and are

displayed continuously.  These are calculated based on the current GMT and

the GMT of launch.

10.2.2 AOS/LOS of TDRS Coverage (Item 2)

The ID and countdown time to AOS or LOS of TDRS coverage is displayed in

this window.  If multiple satellites have overlapping coverage, the AOS

countdown time to the first TDRS will be displayed.  After AOS of the

first TDRS, the ID and LOS of the last TDRS with overlapping coverage is

displayed.  "STAND BY" will be displayed in this window until AOS/LOS

times have been calculated.

10.2.3 AOS/LOS of Ground Communication Site (Item 3)

This window displays the ID, radio frequency band(s), and countdown time

to AOS or LOS of a ground communication site.  AOS and LOS are defined as

an elevation of the Orbiter, viewed from the ground site, of 0.0 degrees.

Neither terrain masking nor atmospheric refraction are considered in this

calculation.  If sites have overlapping coverage, the AOS countdown time

of the first site will be displayed.  After AOS of the first site, the ID

and LOS of the last site in this overlapping coverage is displayed.  Then

the radio frequency band(s) displayed will be that of the currently

acquisitioned site(s).  "STAND BY" will be displayed in this window until

AOS/LOS times have been calculated.

10.2.4 Spacecraft Sunrise, Noon, Sunset (Item 4)

The countdown time to spacecraft sunrise ("SR"), noon ("SN"), or sunset

("SS") is displayed in this window.  If the time to spacecraft

sunrise/sunset or noon is greater than 100 minutes, the timer will

countdown from 99:59 to 99:00 then cycle until 99:59 is the correct time.

Shuttle sunrise and sunset are defined with respect to the upper limb of

the sun with a fixed bias to offset the effects of refraction.  Press

"CODE-S" to toggle between spacecraft sunrise/sunset and noon.  Spacecraft

sunrise, noon, and sunset functions share this window on a mutually

exclusive basis.

10.2.5 TDRS Coverage (Item 5)

Limits of TDRS coverage are displayed assuming average nominal orbital

altitude for a flight.

10.2.6 Earth Day/Night (Item 6)

The sunlight terminator separates day/night regions on earth's surface and

is updated in real time.  The region in daylight is displayed with inverse

video.

10.2.7 Earth Observation Area (Item 7)

The flashing rectangle highlights an earth observation site.  The area

approximates extent of the site to be observed.  A site is acquired when

any part viewed from the Orbiter is within 40 degrees of the current

nadir.  The next available site will initially be displayed.  Press

"CODE-F" to move forward one available site at a time until the last

predicted ground track point is encountered.  To return to the next

available site, press "CODE-R".  The displayed site is shown until

acquired, then the next site is highlighted with a flashing rectangle.

10.2.8 Ground Communication Sites (Item 8)

Limits of selected ground communication sites, with terrain masking, are

displayed assuming average nominal orbital altitude for a flight.

10.2.9 Shuttle Bug (Item 9)

The nose of the flashing Shuttle Bug indicates current position of the

Orbiter.  Every thirty seconds, the Shuttle Bug's position is updated to

the next predicted ground track point.

10.2.10 Enlarged F-Stop Display (Item 10)

For ease of viewing, the current camera F-Stop is enlarged and displayed.

This is the same camera F-Stop value described in Item 13.  The enlarged

F-Stop display may be toggled ON or OFF by pressing "CODE-CTRL-SHIFT-F".

For high inclination orbits, position of the enlarged F-Stop display will

vary slightly.  This is done to avoid display conflicts with predicted

ground tracks and the Shuttle Bug.

10.2.11 Current and Two Predicted Ground Tracks (Item 11)

Predicted ground tracks are updated when the Shuttle Bug reaches the

predicted ground track point 5400 seconds (90 minutes) from the first.

The Shuttle Bug then occupies the first ground track point in a series

extending 4.5 hours into the future.  Adjacent ground track points are

separated in time by 30 seconds.

10.2.12 Tone Indicator (Item 12)

"ON" indicates a tone will be generated when the currently displayed earth

observation site countdown timer reaches two minutes.  "OFF" indicates no

tone will be generated.  The default setting is "ON".  The tone indicator

may be toggled "ON" and "OFF" by pressing "CODE-T".

10.2.13 Latitude, Longitude, and Current F-Stop (Item 13)

The Orbiter's current latitude, longitude, and the camera F-Stop required

for an average albedo target at the current spacecraft nadir are updated

once every 30 seconds.  "CODE-L" immediately updates this display and

toggles it from dynamic to frozen. While frozen, the display will flash

approximately once/second.  This display window is shared, on a mutually

exclusive basis, with items 16, 17, and 18.

10.2.14 Ascending Node Time (Item 14)

This window displays elapsed time since previous ascending node or

countdown to the next ascending node.  The display is controlled in

conjunction with items 16 and 17.

10.2.15 Earth Observation Site (Item 15)

The site ID is displayed along with the countdown time to acquisition of

the displayed earth observation site.  The program searches for

acquisition only.  Once acquisition is achieved, the program will search

for acquisition of the next available site.  If no earth observation site

has been found within the predicted ground tracks, "*****  XX:XX" will be

displayed.  The F-Stop for this site is also displayed.  This F-Stop is

based on a predetermined albedo for the current site, a set film and

shutter speed, and the computed sun angle.  Since F-Stop values are

computed in discrete 1/2 F-Stop steps, a difference of 1/2 F-Stop between

the SPoC value and a premission value is not unusual.

SPoC observation site F-Stops are computed for acquisition, but the

premission albedos consider an average across the entire site.  For sites

with significant physical size, this can result in F-Stop differences of

as much as 1 and 1/2 to 2 full F-Stops in extreme cases.

10.2.16 Longitude, MET, and Countdown Timer For Next Ascending Node (Item

16)

A keyboard input of "CODE-N" will display the longitude and MET of the

next ascending node in the bottom center window, replacing any data

previously displayed there.  Also the countdown time for this node will be

displayed in the Ascending Node Time window (Item 14).  This function

shares the bottom middle window, on a mutually exclusive basis, with Items

13, 17, and 18.

10.2.17 Longitude, MET, and Time Since Previous Ascending Node (Item 17)

A keyboard input of "CODE-CTRL-SHIFT-N" will display the longitude and MET

of the previous ascending node in the bottom center window, replacing any

data previously displayed there.  Elapsed time since the previous

ascending node will also be displayed in the Ascending Node Time window

(Item 14).  This function shares the bottom middle window, on a mutually

exclusive basis, with Items 13, 16, and 18.

10.2.18 Sun Glint Angle and Azimuth (Item 18)

A keyboard input of "CODE-G" will display the sun glintpoint

angle-of-reflection and azimuth relative to the Orbiter.  These data are

displayed in the bottom center window, replacing any data previously

displayed there. "GLINT ANGL" is the angle-of-reflection of the sun's

image off the Earth's surface.  "AZ" is the angle measured from the

vehicle true heading in the Orbiter local horizontal plane to the

glintpoint azimuth and accumulated in the direction "east of north".  In

other words, the difference "true azimuth of the sun glintpoint minus

current Orbiter true heading" is "AZ".  Sun glint data are displayed only

if the "GLINT ANGL" is between 9.99 degrees and 80.21 degrees.  Otherwise,

Xs are displayed in place of the data.  This function shares the bottom

middle window, on a mutually exclusive basis, with Items 13, 16, and 17.

10.2.19 Sun Glintpoint Area (Item 18a)

A "CODE-G" keyboard input will also graphically display the sun glintpoint

relative to the Shuttle by flashing a small square on the WORLD MAP.

10.3 PSAT

A Predicted Site Acquisition Table (PSAT) is available for the current and

two predicted ground tracks.  Press "CODE-P" to call up the PSAT Display.

There are two windows for this display.  The upper window displays TDRS

data, while the lower window displays ground site data.  Each window has a

light bar which serves as an aid for reading data across a row.  Press

"UPARROW" and "DOWNARROW" to control light bar movement in the lower

window.  Press "SHIFT-UPARROW" and "SHIFT-DOWNARROW" to control light bar

movement in the upper window.

If there are more lines of data than can be displayed in a PSAT window at

one time, only sites with LOS in the future will be visible when the

display appears.  Users may scroll up or down to view data not currently

in the window.  Small up/down arrows indicate if more data are available.

Users may scroll in the direction of the arrows to view the additional

data.  When the bottom or top of the table is reached, the appropriate

arrow will vanish.

"More Data Arrows": Indicate that more data are available above or below

those currently visible.

"LOS Indicators": Indicate the site has an LOS in the past.

"Light Bar": Visual aid in reading across a row.  May be moved with the

arrow keys as described above.

"XX/XX:XX:XX    XX:XX": Indicates the data are not currently available.

To return to the WORLD MAP Display, press "CODE-ESC"

10.4 SITE MANAGER

SITE MANAGER allows users to:

* 1) Turn communication sites ON or OFF.

* 2) Add/edit communications site data.

* 3) Delete user input communications sites.

* 4) Add/edit earth observation site data.

* 5) Delete user input earth observation sites.

On the first call to WORLD MAP from the SPoC Main Menu, the SITE MANAGER

Main Menu will be displayed.  During subsequent calls to WORLD MAP,

control will pass directly to the WORLD MAP Display.  SITE MANAGER may

also be invoked from the WORLD MAP Help Menu or by pressing "CODE-E" from

the WORLD MAP Display.

The SITE MANAGER Main Menu is displayed on the first call to WORLD MAP as

a reminder that the previous user may have reconfigured communication or

earth observation sites.  This allows users easy access to verify

configuration of the sites.

If a communication site is ON, the site will be included in AOS/LOS

searches.  If it is OFF, the site will not be included in AOS/LOS

searches.  The AOS/LOS search drives the WORLD MAP AOS/LOS countdown

windows and PSAT Display.  SPoC premission site data cannot be altered or

deleted by users.  Only user input site data may be altered or deleted by

users.

10.4.1 SITE MANAGER Exit Options

Three options on the SITE MANAGER Main Menu will exit SITE MANAGER and

proceed to the WORLD MAP program.

10.4.1.1 CODE-D     INVOKE DEFAULT COMMUNICATION SITES

The communication site list will revert to the premission default

configuration.  Any user input sites marked for deletion will be deleted.

All other sites, including user input sites, will be turned OFF.  SITE

MANAGER is exited and the WORLD MAP Display appears.

10.4.1.2 CODE-S     SAVE CHANGES PROCEED TO DAYMAP

Any changes made during this SITE MANAGER session will be saved.  User

input sites marked for deletion will be deleted.  SITE MANAGER is exited

and the WORLD MAP Display appears.

10.4.1.3 CODE-ESC     NO CHANGES PROCEED TO DAYMAP

Any changes made during this SITE MANAGER session will be ignored.  SITE

MANAGER is exited and the WORLD MAP Display appears.  "NOTE": If changes

were made, users will be warned with the following message:

"PRESS CODE-RETURN TO EXIT SITE MANAGER WITHOUT SAVING CHANGES"

"PRESS CODE-ESC TO RETURN TO SITE MANAGER"

This prevents users from inadvertently losing any changes that may have

been made during this SITE MANAGER session.

10.4.2 SITE MANAGER Functions

10.4.2.1 CODE-N     SELECT COMMUNICATION NETWORK

This option displays the Network form which allows users to configure

entire communication networks "ON" or "OFF".  If "ON" is selected, all

communications sites in that network will be turned "ON".  Otherwise, all

sites for that network will be turned "OFF".

10.4.2.2 CODE-I   SELECT INDIVIDUAL COMMUNICATION SITES

This option displays the Communication Site Selection form.  Users may

configure each communication site "ON" or "OFF".  Sites configured

premission as "ON" are indicated by "DEF" (default) in the fourth column

from the left.  Sites are grouped by site type in alphabetical order.

TDRS satellites are at the top of the form, followed by STDN and Remote

Tracking Station (RTS) sites.  If any user input sites have been added,

they will appear at the bottom of the form.

10.4.2.3 CODE-P     ADD COMMUNICATION SITE

This option allows users to add or edit user input site data.  Site data

loaded premission can not be altered by users.  Only user input site data

may be altered.

The Site ID form appears after "CODE-P" is pressed.  A site name composed

of three alpha characters is the first item to be entered on this form.

In the following examples, "ABC" will be used as the site name.  The

characters entered may be either upper or lower case.

The second Site ID form item allows users to select whether inputs pertain

to a ground site or TDRS.

Acceptable inputs for the Site ID form:

* Site Name : Exactly three alphabetic characters.

Possible error messages:

* 1) "IMPROPER COMMUNICATION SITE ID (REENTER ID)"

* Problem: Three alphabetic characters were not entered for the site ID.

* Solution: Enter three alphabetic characters.

* 2)              "DUPLICATE SITE ID"

*    "IMPROPER COMMUNICATION SITE ID (REENTER ID)"

* Problem: The site ID entered was used for a premission site.

* Solution: Use a different site ID for the input site.

10.4.2.3.1 Adding a Ground Communications Site

If "GROUND SITE" was selected from the Site ID form, the Site Data form

will appear.  At the top of the form, "ENTERING ABC DATA" is displayed.

Also notice that the latitude and longitude settings are blank.  Enter the

latitude and longitude (in decimal degrees).

Select the correct frequency band for this input site with the "LEFTARROW"

and "RIGHTARROW" keys.

10.4.2.3.2 Adding a TDRS

If "TDRSS" was selected from the Site ID form, the Site Data form will

appear as shown in.  At the top of the form, "ENTERING ABC DATA" is

displayed.  Notice that the longitude setting is blank.  Enter the

longitude (in decimal degrees).

10.4.2.3.3 Editing User Input Ground Site Data

If a site has previously been input and "GROUND SITE" was selected from

the Site ID form, the Site Data form will appear as shown in.  At the top

of the form, "EDITING ABC DATA" is displayed.  The form will display site

data for editing.

10.4.2.3.4 Editing User Input TDRS Data

If a TDRS has previously been input and "TDRSS" was selected from the Site

ID form, the Site Data form will appear.  At the top of the form, "EDITING

ABC DATA" is displayed. The TDRS longitude may then be updated.

Acceptable inputs for the Site Data forms:

* Latitude: -90 through +90 (ground sites only)

* Longitude: -180 through +360

Possible error messages:

* 1) "IMPROPER LONGITUDE ( REENTER LONGITUDE )"

*     "IMPROPER LATITUDE ( REENTER LATITUDE )"

* Problem: The inputs are out of range.

* Solution: Correct erroneous inputs.

10.4.2.4 CODE-X     DELETE COMMUNICATION SITES

Only user input communication sites may be deleted.  To mark one or more

sites for deletion, select "CODE-X   DELETE COMMUNICATION SITES" from the

SITE MANAGER Main Menu.  If there are currently any user input sites, the

Communication Site Deletion form will appear.  If there are currently no

user input sites, this message will be displayed:

"THERE ARE NO "INPUT" SITES TO DELETE

PRESS ANY KEY TO CONTINUE

COMMUNICATION SITES"

Communication Sites Deletion Form

Note that sites marked for deletion on this form will only be purged from

bubble memory when "CODE-D" or "CODE-S" is selected from the SITE MANAGER

Main Menu.  Users may return to the Communication Site Deletion form and

mark or unmark sites for deletion on multiple occasions before they are

purged.

User input sites marked for deletion will be accompanied by an asterisk

when displayed in the Site Selection form.

10.4.2.5 CODE-CTRL-SHIFT-P     ADD EARTH OBSERVATION SITE

This option allows users to add or edit a user input earth observation

site.  All user input earth observation sites are point sites.  A point

site is one point on the earth's surface defined by its latitude and

longitude.   Sites that are loaded premission cannot be altered by users.

Only user input site data may be altered.

To add or edit earth observation site data, select "CODE-CTRL-SHIFT-P

ADD EARTH OBSERVATION SITE" from the SITE MANAGER Main Menu.  The

Observation Site ID form will be displayed.  For the following examples,

"T104B" will be used as the site name.

After pressing "CODE-RETURN" in the Observation Site ID form, the

Observation Site Data form  will appear.  The format of the Observation

Site Data form will vary based on the inputs into the Observation Site ID

form.

Acceptable inputs for the Observation Site ID form:

* Site Name : Maximum of five alpha-numeric characters.

Possible error messages:

* 1) "IMPROPER EARTH OBSERVATION SITE ID (REENTER ID)"

* Problem:  Improper site name was entered.

* Solution: Reenter site name.

* 2)              "DUPLICATE SITE NAME"

*    "IMPROPER EARTH OBSERVATION SITE ID (REENTER ID)"

* Problem: The site name entered was used for a premission site.

* Solution: Use a different site name for the input site.

10.4.2.5.1 Adding An Earth Observation Site

If this is a new site to be added, "ENTERING T104B DATA" is displayed at

the top of the Observation Site Data form, indicating that data for the

new site "T104B" are being entered.  Since this is a new site, the form

settings are blank.  Enter the setting for each item.

10.4.2.5.2 Editing User Input Earth Observation Site Data

If the site name entered into the Site ID form has previously been added,

the Observation Site Data form will be displayed with "EDITING T104B DATA"

at the top of the form.  The data for this site will be displayed in the

form.  Edit the necessary settings.

Acceptable inputs for the Site Data form:

* Latitude:  -90 through +90

* Longitude: -180 through +360

* Albedo:     6 through 18

Possible error messages:

* 1) "IMPROPER LONGITUDE ( REENTER LONGITUDE )"

*    "IMPROPER LATITUDE ( REENTER LATITUDE )"

*    "IMPROPER ALBEDO (REENTER ALBEDO)"

* Problem: The inputs are out of range.

* Solution: Correct erroneous inputs.

10.4.2.6 CODE-CTRL-SHIFT-X     DELETE EARTH OBSERVATION SITES

Only user input earth observation sites may be deleted.  To mark one or

more sites for deletion select "CODE-CTRL-SHIFT-X   DELETE EARTH

OBSERVATION SITES" from the SITE MANAGER Main Menu.  If there are

currently any user input sites, the Observation  Site Deletion form will

appear.  If there are currently no user input sites, this message will be

displayed:

"THERE ARE NO "INPUT" SITES TO DELETE

PRESS ANY KEY TO CONTINUE

OBSERVATION SITES"

Note that sites marked for deletion on this form will only be purged from

bubble memory when "CODE-D" or "CODE-S" is selected from the SITE MANAGER

Main Menu.  Users may return to the Earth Observation Site Deletion form

and mark or unmark sites for deletion on multiple occasions before they

are purged.

APPENDIX A--INITIALIZING THE SPoC

The following set of instructions leads a user through the procedures to

initialize the SPoC.  After SPoC initialization is completed, the user may

proceed to APPENDIX B (WORLD MAP), C (DEORBIT), or D (CGMANAGER) for a

demonstration of SPoC applications.  Familiarity with basic SPoC operation

is assumed.  For the user unfamiliar with the SPoC, hardware setup

procedures for ground operation are found in Section 2.2.  Details of

basic SPoC operations are contained in Section 3.

When the SPoC is turned on, the Boot Up Screen is displayed after several

seconds. After the operating system has been loaded, the Boot Up Screen

will be erased.

Messages will then be displayed informing users that the SPoC EXECUTIVE is

searching for programs and data files.

Upon completion of the file search, the first initialization form will be

displayed (LAUNCH TIME UPDATE form).  For this demo, follow the steps

below to enter the launch date/time, the current SGMT, and state vector.

"LAUNCH TIME UPDATE Form"

1)  Press "CODE-BACK SPACE" to erase old data.

YEAR: enter "1988"

press "RETURN".

2)  Press "CODE-BACK SPACE" to erase old data.

DAY: enter "300"

press "RETURN".

3)  Press "CODE-BACK SPACE" to erase old data.

HOUR: enter "0"

press "RETURN".

4)  Press "CODE-BACK SPACE" to erase old data.

MINUTE: enter "0"

press "RETURN".

5)  Press "CODE-BACK SPACE" to erase old data.

SECOND: enter "0"

press "CODE-RETURN".

"GMT UPDATE Form"

6)  Press "CODE-BACK SPACE" to erase old data.

DAY (GMT): enter "300"

press "RETURN".

7)  Press "CODE-BACK SPACE" to erase old data.

HOUR (GMT): enter "0"

press "RETURN".

8)  Press "CODE-BACK SPACE" to erase old data.

MINUTE (GMT): enter "0"

press "RETURN".

9)  Press "CODE-BACK SPACE" to erase old data.

YEAR: enter "1988"

press "CODE-RETURN".

"NOTE:" If a warning message appears at this point, correct bad data and

press "CODE-RETURN".

"STATE VECTOR UPDATE Form"

10) Press "CODE-BACK SPACE" to erase old data.

DAY (GMT): enter "300"

press "RETURN".

11) Press "CODE-BACK SPACE" to erase old data.

HOUR (GMT): enter "0"

press "RETURN".

12) Press "CODE-BACK SPACE" to erase old data.

MINUTE (GMT): enter "0"

press "RETURN".

13) Press "CODE-BACK SPACE" to erase old data.

SECOND (GMT): enter "0"

press "RETURN".

14) Press "CODE-BACK SPACE" to erase old data.

X POSITION (KFt): enter "-4065.5451"

press "RETURN".

15) Press "CODE-BACK SPACE" to erase old data.

Y POSITION (KFt): enter "19478.6516"

press "RETURN".

16) Press "CODE-BACK SPACE" to erase old data.

Z POSITION (KFt): enter "-9134.2843"

press "RETURN".

17) Press "CODE-BACK SPACE" to erase old data.

X VELOCITY (KFt/SEC): enter "-23.4998543"

press "RETURN".

18) Press "CODE-BACK SPACE" to erase old data.

Y VELOCITY (KFt/SEC): enter "-7.6119311"

press "RETURN".

19) Press "CODE-BACK SPACE" to erase old data.

Z VELOCITY (KFt/SEC): enter "-5.7686384"

press "CODE-RETURN".

At the bottom of the screen the inclination, HA, and HP are displayed.

This serves as a data check for state vector inputs.  For the above

inputs, the following values will be displayed.

INCLINATION = 28.518

APPROX. ORBIT : HA=166; HP=159

20) Press "CODE-RETURN".

The SPoC Main Menu will now be displayed.  The user may select SPoC

applications for execution.  Step-by-step demos are available for WORLD

MAP (Appendix B), DEORBIT (Appendix C), and CGMANAGER (Appendix D).  The

STATE VECTOR UPDATE and SET GMT programs are also available from the SPoC

Main Menu, and were demonstated during the initialization procedures.

You are now ready to proceed to a SPoC demo of your choice.

APPENDIX B--WORLD MAP DEMONSTRATION

Before exercising WORLD MAP demonstration procedures, the user should

complete procedures outlined in APPENDIX A (Initializing The SPoC).  The

SPoC Main Menu should be currently displayed.  The following instructions

will lead the user through a WORLD MAP demo.

1) Select "DISPLAY WORLD MAP" from the SPoC Main Menu.

The first time WORLD MAP is selected from the SPoC Main Menu, the SITE

MANAGER Main Menu will be displayed.  SITE MANAGER is a WORLD MAP function

which may be exercised later during this demo.  For now, continue on to

the WORLD MAP display.

2) Select "INVOKE DEFAULT COMMUNICATION SITES" from the SITE MANAGER Main

Menu.

After a few seconds, the WORLD MAP Display will appear.  Notice that the

communication site AOS/LOS windows display the words "STAND BY".

As soon as the AOS/LOS search has been completed, windows will display

AOS/LOS countdown times.

3) Press "CODE-?" to display the WORLD MAP Help Menu.

This menu provides the "CODE-[key]" combinations associated with each

WORLD MAP function.  WORLD MAP functions may be invoked from the Help

Menu.  They may be invoked from the WORLD MAP display directly with the

proper "CODE-[key]" input.

4) To return to the WORLD MAP Display, press "CODE-ESC".

At this time, you may want to explore various WORLD MAP functions.  For a

detailed explanation of each WORLD MAP function, refer to Section 10.2.

Section 10.3 details the PSAT Display, and Section 10.4 details SITE

MANAGER functions.

5) When you are ready to exit WORLD MAP and return to the SPoC Main Menu,

press "CODE-ESC".

APPENDIX E--SPoC HISTORY

The concept of an independent computational capability aboard the Shuttle

has been operational since STS-1.  Originally, an HP41 calculator was

employed for this purpose.  Data storage and manipulation capabilities,

absent from the GPC's because of core storage limitations, were provided

by this hand-held calculator.  Also provided by the HP41 were capabilities

similar to existing ground-based processors in the event that the Shuttle

and crew were in a no-communication situation.  As of April 5, 1983 the

HP41 flight programs included the following:

a) The CG program allowed the crew to track current X and Y-CG location as

a function of MET, OMS, and RCS propellant loadings, and whether payloads

were berthed or deployed.

b) The AOS program supplied the crew with information concerning the next

time the spacecraft would acquire a communications site.

c) The LANDTRACK program provided observation pass information for any

stored earth observation site.

d) The DEORBIT program provided the crew with deorbit maneuver targets in

the event of total loss of communications with the ground and a

time-critical systems failure requiring an emergency deorbit.

e) The ORBIT program provided the crew with data concerning attitudes and

rates involved in the star sighting function and with orbit sunrise and

sunset.

f) The ALARM program gave the crew a personalized reminder of upcoming

events of particular interest by having the calculator sound a tone and

display a pre-stored message.

g) The LANDING program computed data necessary for the approach and

landing phase of flight.

Although the HP41 has proven to be a useful tool, it has a very limited

memory capacity (only one of the above programs can be stored in memory at

a time), very limited text displays, and there is no graphics capability.

As new requirements developed for independent computational capabilities

aboard the Shuttle, the HP41 became inadequate for the task.  To augment

the hand-held calculator, the SPoC was introduced.  SPoC is an expansion

of the HP41 calculator concept and enhances independent computation and

display capabilities by providing more memory, more mass storage, and

graphic displays. In 1983, the Ascent/Entry Procedures Section of the

Operations Branch began evaluating microcomputers for onboard

implementation.  After evaluating several machines, a GRiD microcomputer

was selected because it met the hardware and software requirements and

constraints fully.  A proposal was made to the Government Furnished

Equipment (GFE) Configuration Control Board (CCB) to obtain the necessary

funding for the project.  The GFE CCB denied the request.  To gain

necessary support for the project, presentations were made at the branch,

division, and directorate levels.  After a SPoC presentation was made to

the Program Office Manager, Glynn Lunny, approval for the SPoC project was

given. Proposals were again made to the GFE CCB, and on 08/15/83 a Crew

Related GFE CCB Directive (#3D149) was issued authorizing purchase,

testing, certification, and preparation for flight of a GRiD Compass

Computer on STS-9.

Purchased commercially and modified, the original SPoC was a GRiD Compass

1101 lap top computer.  The Compass 1101 has a 4.75 by 3.5 inch

electroluminescent screen, 384K bytes of non-volatile bubble memory and

256K bytes of RAM. Modifications included replacing the internal modem

with a fan for cooling, since the standard Model 1101 computer uses

convective cooling.  The lithium battery circuit, which provides power for

the internal clock while the Compass is not connected to an external power

supply, had supplemental diodes and a fuse installed to insure failsafe

operation. A power cable was designed for Orbiter power connector

compatibility.  EMI tests showed the SPoC was out of specification.  Since

the out-of-spec condition for SPoC would cause no problems for Orbiter

systems, communications, or payloads, the Space Shuttle Program

Requirements Control Board issued a directive (#S23562) to authorize a

waiver permitting the radiated EMI out-of-spec condition.

SPoC was initially flown on STS-9 as a proof of concept engineering

evaluation.  During this flight, it proved useful as an aid to crew

awareness and productivity.  SPoC flight software for STS-9 included

programs that calculated and/or displayed the following data:

MET

The world map (facsimile of the MCC world map)

Day and night areas on the earth's surface

AOS/LOS ID of the current/next ground communication site

AOS/LOS and ID of current/next TDRSS coverage

Current and two predicted ground tracks

Current position of the Orbiter on the world map

AOS of the next earth observation site

Brewster Shaw, pilot for STS-9, stated "It's really a big asset to know

where we are all the time."  Since SPoC was such an asset on STS-9, Flight

Equipment Project Office Configuration Control Board Directive #G1309

authorized one SPoC and power cord be manifested for STS-11 and subsequent

missions.

An upgrade occurred in late 1984, with the Compass 1129 replacing the 1101

for STS-51C and subsequent missions.  The GRiD Compass 1129 is the same

size as the 1101, with a 4.75 by 3.5 inch electroluminescent screen and

384K bytes of non-volatile bubble memory.  Improvements gained with the

1129 include twice as much RAM (512K bytes) and four accessible slots for

PROM or EPROM chips.  At this time, the necessary hardware and software

were purchased to develop EPROMs for flight.  Use of socketed PROM and

EPROM chips was viewed as a possible interim solution for expanded mass

memory until a mass memory device was certified for flight.  Compass 1129s

were purchased commercially and modified.  The internal modem was replaced

with a fan for cooling and an EMI reducing "Tempest" screen was installed.

For crew safety, the lithium battery which provided power for the internal

clock was exchanged for a replaceable silver oxide battery and a clock

reset circuit.  Also the foot was milled out to promote internal

ventilation while in the closed position for flight operation.

In 1985, the SPoC was upgraded to a GRiD Compass 1139 for STS-51G and

subsequent missions.  The GRiD Compass 1139 is similar to the 1129, having

the same memory capacity.  It is an improvement over the 1129 because it

has a larger electroluminescent screen (7.5 by 3.5 inches) and a standard

internal fan for cooling. Like the 1129, this unit was purchased

commercially and modified for flight.  Modifications include an

EMI-reducing "Tempest" screen, a replaceable silver oxide battery, a clock

reset circuit, a milled out foot, and the modem is removed.

The original flight software configuration has been augmented since STS-9

to satisfy expanding functional requirements levied on the SPoC.  The

flight software configuration now includes programs that calculate and/or

display the following data:

Predicted Site Acquisition Table

Table of TDRSS AOS/LOS

Communication/Earth Obs site reconfiguration

Latitude and longitude of the Orbiter

AOS of future earth observation sites

Time to Orbiter sunrise, noon, sunset

Current F stop

F stop for the selected earth observation site

Sun glint position on the world map

Sun glint look angles

Longitude and MET of the next/previous ascending node

Time until/from the next/previous ascending node

Deorbit opportunities

Deorbit maneuver targeting

Orbiter CG and weight

CG management procedures

The SPoC has also been used to provide computational support for special

purpose applications on a flight-specific basis.  The Orbital Refueling

System (ORS) and the Gravity Gradient Free Drift experiments set a

precedent for using microprocessors  to support payload experiments.

Since 41-G, two SPoCs have been manifested for each Shuttle mission.  The

primary unit is usually positioned on the forward flight deck above the

eyebrow panel, while the backup nominally supports middeck operations.

Even during emergency power-downs, one of these units remains running.

"NONCRITICAL BUT MANDATORY"

Although the SPoC was classified as noncritical hardware by Configuration

Control Board Directive 1253, it is considered mandatory equipment.  If

the SPoC fails, neither the crew nor the mission is threatened: thus the

noncritical classification.  However, the SPoC is such an asset to the

crew and to MCC that it is mandatory equipment for flight.

The following excerpts from reports and memos illustrate the importance of

the SPoC to Shuttle missions.

"STS-9 Flight Crew Report" (Dec. 1983): "The SPoC proved to be a great

asset.  It was unstowed and set up during the post insertion timeframe and

remained active until deorbit prep... The SPoC allowed the crew to see

where they were at all times and served as an accurate source of

communications coverage information.  In light of the many dropouts of

TDRSS S-Band signal that were experienced, the SPoC AOS/LOS information

allowed the crew to manage the comm system in an efficient manner and

eliminated unnecessary trouble shooting.  The SPoC appears to have great

potential that deserves further development for future Shuttle missions."

"Memo from the Space Shuttle Earth Observations Project Manager"

(03/02/84, #SC5-84-32):  "The GRiD SPoC has been of great value to the

Space Shuttle Earth Observations Project... Our contact with the crews

that have used the SPoC ...reflect extreme interest in its present

capabilities and proposed updates."

The value of the SPoC in monitoring a payload was demonstrated for the

STS-41G ORS test:

"Memo from John Young" (04/18/84, #CB-84-042):  "The SPoC has been

determined by the crew to be not just useful, but mandatory for proper

crew monitoring of ORS hydrazine transfer operations.  It provides the

proper displays that provide the crew their only insight into the

hydrazine heat transfer occurring during an ORS transfer operation for all

initial conditions.  Using the SPoC information and the other ORS displays

and controls, timely and proper response can be made by the crew to any

ORS anomaly."

"STS-41G Debrief Notes" (10/18/84): "[The SPoC was] very helpful because

no one had time to just sit by a window and watch for sites and

phenomena."

"STS-51C Crew Report": "The presence of SPoC on our mission was a major

asset in helping us perform many of our tasks.  Future capabilities for

SPoC will increase its utility even more...these computers have proven to

be very reliable..."

"Memo from On-Orbit Flight Techniques Panel" (01/25/88, #DA8- 88-04):

"After reviewing the functions provided by the SPoC and assessing the

significant degree of support it provides for routine flight crew

operations, the FTP considers it to be a very important part of crew

equipment and recommends the Program consider it to be 'mandatory' for all

STS flights.

"The FTP also recommends that the SPoC should continue to be classified as

criticality 3.  These recommendations are based on two considerations.

a) The primary SPoC function, that of an orbit trajectory status display,

does not jeopardize the crew, vehicle, or mission in the event of

malfunction.

b) The secondary SPoC function of providing a backup deorbit targeting

capability for emergency deorbit would only be used when the Mission

Control Center is unavailable for Orbiter support and an immediate deorbit

was required.  The probability of this happening is extremely remote

because for every case except a large cabin leak, it takes multiple

failures before this capability would be needed, and most of those

combinations of failures, such as failure of both freon loops, are

considered non-credible.

"Retaining the SPoC's criticality 3 classification is highly desirable

because it allows a flexible and rapid response to new requirements.  To

retain these desirable characteristics, functions considered inappropriate

for this classification should be integrated into other processors of an

appropriate classification."

APPENDIX F--GLOSSARY

AOS: Acquisition of Signal

Boot Cycle: Period during which the computer is loading the operating

system and SPoC EXECUTIVE software.

CG: Center of Gravity

Choice Band: Rectangular area at the top of a form which will contain

choices or pertinent information.

Choices: Predefined values for a setting.

Command Key: RETURN key, ESCAPE key, and any other keys that select a menu

option.

Confirm: Keyboard input of CODE-RETURN.

Edit Prompt: A small triangular symbol which indicates a user input is

expected.

EMCC: Emergency Mission Control Center

EMI: Electromagnetic Interference

EPROM: Erasable Programmable Read Only Memory

FDO: Flight Dynamics Officer

Form: User interface display which allows the user to enter or edit data

for one or more items.

FRCS: Forward Reaction Control System

GFE CCB: Government Furnished Equipment Configuration Control Board

GMT: Greenwich Mean Time

GNC: Guidance Navigation and Control

GPC: General Purpose Computer

Highlighted Box: If choices are available in the choice band, the

highlighted box indicates the current choice or highlights pertinent

information.

ISP: Specific Impulse

Item: Label which identifies the data to be modified in a form.

LOS: Loss of Signal

LOT: Landing Opportunities Table

MCC: Mission Control Center

Menu: User interface display which allows the user to select one of the

available options.

MET: Mission Elapsed Time

MOC: Mission Operations Computer

MOCR: Mission Operations Control Room

Modifier Keys: CODE keys, SHIFT key, and CONTROL key.

Nadir: Projection of the Orbiter's position along the local vertical onto

the Earth's surface.

OMS: Orbital Maneuvering System

ORS: Orbital Refueling System

PROM: Programmable Read Only Memory

PSAT: Predicted Site Acquisition Table

Prompt: Message informing the user what action should be taken next.

PTI: Program Test Input

RAM: Random Access Memory

RCS: Reaction Control System

RMS: Remote Manipulator System

ROM: Read Only Memory

RTS: Remote Tracking Station

Selection Box: Rectangular outline enclosing the current item in a form or

the current option in a menu.

Setting: Data value in a form.

SGMT: Simulated GMT

SPEC: GPC display used to view and modify parameters.

SPoC: Shuttle Portable Computer

STDN: Spaceflight Tracking and Data Network

TDR: True of Date Rotating

TDRS: Tracking and Data Relay Satellite

TDRSS: Tracking and Data Relay Satellite System

TIG: Time of Ignition